|  | Questions are followed by answer fields. Use the ‘Tab’ key to navigate through. Replace Y/N or Yes/No fields with your answer. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GD-CS-016C |  |  |  |  |  |  |  |  |
| This application should be read in conjunction with the departments GD-CS-016 Street Lighting Guideline. Completed applications should be sent to assetinformation.dli@nt.gov.au Note: This datasheet is for DLI only. The vendor is to fill the blank sections in the datasheet. The values shown are according to the current standards AS/NZS 1158. The vendor should state how many spare luminaires and parts can be stored. |  |  |  |  |  |  |  |  |
| Vendor Details |  |  |  |  |  |  |  |  |
| Vendor |  |  |  |  |  |  |  |  |
| Vendor Contact |  |  |  |  |  |  |  |  |
| Email |  |  |  |  |  |  | Phone |  |
| Luminaire Datasheet |  |  |  |  |  |  |  |  |
| Luminaire make |  |  |  |  |  |  |  |  |
| Luminaire model |  |  |  |  |  |  |  |  |
| Luminaire weight (kg) |  |  |  |  |  |  |  |  |
| LED chip make |  |  |  |  |  |  |  |  |
| LED chip model |  |  |  |  |  |  |  |  |
| DALI dimmable drive make |  |  |  |  |  |  |  |  |
| DALI dimmable drive model |  |  |  |  |  |  |  |  |
| Driver configuration DC output current supplied directly to each LED group (mA) |  |  |  |  |  |  |  |  |
| Luminaire output (lumens) |  |  |  |  |  |  |  |  |
| Luminaire input power (watts) |  |  |  |  |  |  |  |  |
| Total luminaire input power (watts) Must be ≤100W |  |  |  |  |  |  |  |  |
| Upward Tilt angle must be 0 or 5 (degrees) |  |  |  |  |  |  |  |  |
| Maintenance factor Must be ≤ 0.8 |  |  |  |  |  |  |  |  |
| Average carriageway luminance (cd/m2) Range is between 0.35 – 1.50 Depending on the subcategory V |  |  |  |  |  |  |  |  |
| Overall uniformity (min/avg) Must be ≥ 0.33 |  |  |  |  |  |  |  |  |
| Longitudinal uniformity (min/max) Must be ≥ 0.5 |  |  |  |  |  |  |  |  |
| Upward waste light ratio Must be ≤ 3% |  |  |  |  |  |  |  |  |
| Threshold Increment (TI) must be ≤ 15% |  |  |  |  |  |  |  |  |
| Surround verge illuminance (ESL and ESR) ≥ 50% |  |  |  |  |  |  |  |  |
| Checklist |  |  |  | Yes | No | Comments |  |  |
| Integral tilt range -10 to 0 |  |  |  | ☐ | ☐ |  |  |  |
| National Electrical Manufacturers Association (NEMA) 7-pin base photoelectric (PE) cell |  |  |  | ☐ | ☐ |  |  |  |
| NEMA socket (ANSC136.41) Receptable (on top) |  |  |  | ☐ | ☐ |  |  |  |
| Required documents attached |  |  |  | ☐ | ☐ |  |  |  |
| Perfect lite printouts |  |  |  | ☐ | ☐ |  |  |  |
| Agi32 printouts |  |  |  | ☐ | ☐ |  |  |  |
| Agi32 *.AGI file attached |  |  |  | ☐ | ☐ |  |  |  |
| LM-80-08 full report attached |  |  |  | ☐ | ☐ |  |  |  |
| LM-79-08 full report |  |  |  | ☐ | ☐ |  |  |  |
| IES and CIE electronic files |  |  |  | ☐ | ☐ |  |  |  |
| ISTMT report |  |  |  | ☐ | ☐ |  |  |  |
| TM-21-11 calculations |  |  |  | ☐ | ☐ |  |  |  |
| RFP MF calculations |  |  |  | ☐ | ☐ |  |  |  |
| Complying Wieland plug and lead of correct length |  |  |  | ☐ | ☐ |  |  |  |
| Access to fixing bolts |  |  |  | ☐ | ☐ |  |  |  |
| Authorised Officer Details |  |  |  |  |  |  |  |  |
| Authorised Officer |  |  |  |  |  |  |  |  |
| Email |  |  |  |  |  |  | Phone |  |
| Signature |  |  |  |  |  |  | Date |  |
| Vendor Signature |  |  |  |  |  |  |  |  |
| Vendor Contact |  |  |  |  |  |  |  |  |
| Signature |  |  |  |  |  |  | Date |  |

| End of form |  |  |  |
| --- | --- | --- | --- |
| End of form |  |  |  |
| Internal use only - Assessing officer |  |  |  |
| Recommended or not recommended |  |  |  |
| Name |  | Position |  |
| Signature |  | Date |  |
| Delegate |  |  |  |
| Approved or not approved |  |  |  |
| Signature |  | Date |  |
| Organisation advised | Yes / No | Date |  |
| End of form |  |  |  |