project check lists road works

use in conjunction with the

current standard specification

for road works

INTRODUCTION

It is difficult during a visit to a construction site to memorise all of the details that need to be checked and while the obvious ones stand out, many other items that should be checked are forgotten. Supervisors may get distracted or over extended due to time constraints and overlook items that may have more serious consequences later in the project.

These Check Lists provide a guide to the inspections that are required on site for a particular project. They are a précis of the full clauses in the project specification and can be referred back to the specification if the full text of the particular item is required. The Check Lists will remind you of virtually every potential problem in materials, workmanship and coordination between trades.

Project specific checks may be added to the Check Lists for individual projects and the experience of Supervisors can be shared by the addition of particular checks on items or operations that are generally of concern on site. In this case less experienced Supervisors can use these Check Lists to provide them with the back-up of more experienced officers.

Provide one copy of the Check Lists book to the Supervisor and one copy to the Contractor for the particular project.

HOW TO USE THESE CHECK LISTS

Take photocopies of the relevant Check List sections before going on site and study these.

Mark any items in the Check List that are of particular concern.

At the project site, check off the work against the Check Lists and make notes of any action required in the space provided.

When the work is completed and the final check has been made sign off and date in the box provided. There is also space to provide comments.

Keep a record of all of the completed Check Lists for future reference if required. These could prove to be valuable if disputes arise on the project.

There is no better way to document actual site conditions or details than by photographs, so don’t forget to take a camera on site and take images to go with particular items in the Check Lists.

HOLD POINTS AND WITNESS POINTS SCHEDULES

Read these Check Lists in conjunction with the schedules of Hold Points and Witness points. Hold Points and Witness Points are scheduled in a separate document accessible via:

https://transport.nt.gov.au/industry/technical-standards-guidelines-and-specifications/technical-specifications/roads

Project Specific Requirements (PSRs) should also be read in conjunction with this document.

FURTHER DEVELOPMENT

If you have any suggestions to improve on these Check Lists or the system in general please contact

Specification Services.

3rd Floor Highway House, Palmerston.

Email: Specification.Services@nt.gov.au

Comments from in-house staff, consultants or contractors are most welcome.

| Project Title |  |
| --- | --- |
| RFT Number |  |

PROJECT CHECKLISTS

ROAD WORKS

CONTENTS

SECTION

- Miscellaneous Provisions

- Provision For Traffic

- Clearing Grubbing and Rehabilitation

- Earthworks

- Conformance Testing

- Pavements and Shoulders

- Stabilisation

- Spray Sealing

- Dense Graded Asphalt

- Slurry Surfacing

- Miscellaneous Concrete

- Drainage Works

- Protection Works

- Road Furniture and Traffic Control Devices

- Pavement Marking

- Landscape

- Ducting and Conduits

- Traffic Control Signals and Intelligent Transport Systems

- Traffic Counting Stations

- Street Lighting

- Directional Boring

- Protective Coatings

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – MISCELLANEOUS PROVISIONS

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| DISPOSAL OF WASTE |  |
| Dispose of waste of all types in a legal manner at a legal waste disposal site |  |
| Permits – Waste disposal site |  |
| ESTABLISHMENT |  |
| Demobilisation – site restored |  |
| VOLATILE SUBSTANCES MANAGEMENT |  |
| Volatile Substance Abuse Management Plan(s) |  |
| PERMITS TO ACCESS LAND FOR WORKS ON ROADS |  |
| Permissions to access the land where the works are to be carried out |  |
| Comply with all imposed conditions |  |
| CAMP SITES/COMPOUND/WORKSHOP |  |
| Facilities maintained in good condition |  |
| Camp food preparation area |  |
| Waste materials |  |
| On completion, remove facilities and restore site |  |
| EXTRACTION AREAS AND WATER SOURCES |  |
| Approvals obtained – Refer to the Standard Specification for Environmental Management |  |
| Crushing or screening: - Plant certified as fit for use |  |
| Operation of extraction areas: - Only one access road - Adequate drainage maintained |  |
| Extraction requirements complied with |  |
| Excavation limits complied with |  |
| Rehabilitation completed as specified |  |
| Stream sites: - Department of Environment and Natural Resources (DENR) contacted before any work commenced - Excavation limits and conditions complied with |  |
| Allow inspections from DENR at any time |  |
| Provide records as specified |  |
| EXPLOSIVES |  |
| Evidence of licences etc. (required by NT WorkSafe) to Superintendent |  |
| Inspection of structures and services recorded |  |
| PLANT AND EQUIPMENT |  |
| Inspected and maintained |  |
| Cleaning |  |
| Mobile machinery fitted with audible warning alarms, and warning beacons visible and working |  |
| SAFETY |  |
| Safety Officer capable and available at all times |  |
| Work Health and Safety Management Plan |  |
| Safety equipment, protective clothing and devices |  |
| First aid facilities |  |
| Employees instructed on safety hazards and how to avoid injury |  |
| Safety equipment, clothing, and devices used comply with cited Australian Standards |  |
| WORK INVOLVING CHEMICALS |  |
| Safety Data Sheets |  |
| Spill clean-up equipment |  |
| Storage |  |
| FENCING AND SHORING OF OPEN EXCAVATIONS |  |
| Maintain excavation and shoring |  |
| Support trenches in saturated or unstable ground |  |
| WORK ON RAILWAY SITES |  |
| Approval of the owner and accredited rail operator has been obtained |  |
| Approvals or licences when working within 100 m of the Railway |  |
| 14 days’ notice provided of intent to commence work together with work plan |  |
| If work within 3m of rail line work must be co-ordinated through Superintendent |  |
| PROJECT NOTICE BOARDS |  |
| Correct design and wording |  |
| Removed as required at completion |  |
| LEVEL CHECKING |  |
| Checking carried out as per specification |  |
| Independent surveyor used |  |
| Survey pegs installed as per specification |  |
| Audit by Superintendent if required |  |
| CONTROL STATION CHECK SURVEY |  |
| As per Surveys and Setting Out clause in Preliminaries |  |
| CYCLE AND PEDESTRIAN SHARED PATHS |  |
| Concrete shared paths as per specification and cited Australian Standards |  |
| Design complies with AUSTROADS design principles |  |
| AS CONSTRUCTED INFORMATION |  |
| Procedure for recording and submitting the amended drawings |  |
| Design levels and constructed levels are captured |  |
| To NTG Technical Drawings Part 1 - Requirements for Technical Records Management |  |
| PDF and CAD format |  |
| Independent surveyor used |  |
| DIVING WORK |  |
| Comply with Work Health and Safety Act and Regulations |  |
| Comply with ADAS Operation Manual or DRDC Diving Manual or NOAA Diving Manual |  |
| Evidence of competency for all diving personnel |  |
| Dive Safety Log |  |
| Dive Plan |  |
| Crocodile Hazard Management Plan |  |
| Dive cage |  |
| Crocodile net |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – PROVISION FOR TRAFFIC

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - initial & date |
| --- | --- |
| OWNERSHIP MARKINGS ON TEMPORARY TRAFFIC CONTROL SIGNS AND DEVICES |  |
| Limitations as specified |  |
| CLEAN UP OF TRACKED MATERIALS |  |
| Workzone free from dirt, mud, and other detritus material |  |
| WORKZONE TRAFFIC MANAGEMENT |  |
| Traffic management personnel - NT Construction Induction White Card |  |
| Workzone traffic management plan designer (WZ1) - Prerequisites as specified |  |
| Workzone traffic controller (WZ2) - Prerequisites as specified |  |
| Workzone traffic controller (WZ3) - Prerequisites as specified |  |
| Escort mobile works (WZ 4) - Prerequisites as specified |  |
| Trainee traffic controller - Criteria as specified |  |
| Traffic escort vehicle fitted with rotating beacon light or LED equivalent |  |
| Signs and devices not in use: - Covered or removed from the Workzone - Workzone tidy and uncluttered |  |
| Reinstatement of signs and devices after severe weather |  |
| TRAFFIC MANAGEMENT PLAN |  |
| TMP to AS 1742.3 |  |
| TMP document control procedure |  |
| Submission of TMP as specified |  |
| Submission of traffic guidance schemes as specified |  |
| COMPLIANCE CHECKS OF WORKSITE TRAFFIC MANAGEMENT |  |
| Checks as specified |  |
| Appropriate personnel Workzone accreditation |  |
| Check for new worksite hazards |  |
| TRAFFIC MANAGEMENT AUDIT REQUIREMENTS |  |
| Panel period audit consultants names and details given to contractor |  |
| Audits comply to AS 1742.3, AGRS Part 6 and NT legislation |  |
| Suitability audit requirements as specified |  |
| Compliance audit requirements as specified |  |
| Independent third party audits as specified |  |
| Notice of implementation as specified |  |
| Notice of independent third party audits as specified |  |
| CONFORMANCE AUDITING RESULTS |  |
| Audits received within scheduled times |  |
| Monthly reports received |  |
| Lot register received |  |
| AMENDMENTS TO TRAFFIC MANAGEMENT PLANS |  |
| Modifications to TMP received and clearly marked |  |
| Modifications to Risk Assessment received |  |
| WORK IN RURAL AREAS |  |
| Approval required for night works |  |
| WORK IN BUILT UP AREAS |  |
| Approval required for work during restricted hours |  |
| Motor Vehicle Registry permits vehicles over 19 m |  |
| Traffic lanes open as specified |  |
| Lane closures permission required for bullnose or V type barriers |  |
| NIGHT ILLUMINATION |  |
| Illumination to 10 lux |  |
| WARNING DEVICES |  |
| Warning signs must not intrude into sight lines, especially at intersections |  |
| Warning signs not obscured by vegetation |  |
| Works in progress signage as specified |  |
| Multi panel signs as specified |  |
| Positioned correctly |  |
| Correct size and details as specified |  |
| No multi message signs on site |  |
| NT SPECIFIC REQUIREMENTS FOR ROAD WORK SIGNS |  |
| Sign erection as specified |  |
| Advance warning signs as specified |  |
| Star pickets and fence droppers as specified |  |
| Portable Variable Message Signs (VMS) as specified |  |
| Work zone speed limits – Mandatory, as specified |  |
| Temporary speed limits as specified |  |
| Road safety barriers as specified |  |
| Covering of signs as specified |  |
| EXCAVATIONS, STOCKPILES AND GRADIENTS WITHIN WORK ZONES AND CLEAR ZONES |  |
| Shoring to all trenching or excavations which are deeper than 1.5 metres |  |
| Engineers certification where shoring is not present |  |
| Comply with the provisions of the Code of Practice for Excavation Works |  |
| Comply with the NT Work Safe Codes of Practice |  |
| Comply with Safe Work Australia Codes of Practice |  |
| REQUIREMENTS FOR EXCAVATIONS, STOCKPILES OR OTHER GRADIENTS |  |
| Complies with AS 1742.3 |  |
| Complies with Road Safety Barriers clause |  |
| Containment fencing: - Complies with AS 1742.3 - Retroreflective markers fixed in place as specified |  |
| TEMPORARY PAVEMENT MARKING |  |
| Complies with AS 1742.3 |  |
| Removal as specified |  |
| DETOURS, SIDE TRACKS, AND CROSSOVERS |  |
| Construction as specified |  |
| 5 days notice required before opening any side track, detour, or crossover, to traffic |  |
| Maintenance as specified |  |
| Dust suppression |  |
| Removal of debris and rubbish |  |
| ACCESS TO ADJACENT PROPERTIES AND SIDE ROADS |  |
| Access provided |  |
| Signs in place |  |
| Maintenance of access points |  |
| TEMPORARY PEDESTRIAN ACCESS |  |
| Conform to: AS 1742.9 and AS 1742.10 |  |
| TEMPORARY BRIDGING |  |
| Conform to AUSTROADS Guide to Bridge Technology |  |
| Signage, fencing, road safety barriers and or guard railing in place |  |
| CONTRACTORS PLANT AND EQUIPMENT |  |
| Parking and material storage clear of trafficked areas and clear zones |  |
| Floodlights installed as specified |  |
| Alarms installed as specified |  |
| Rotating beacons installed as specified |  |
| Registration details |  |
| ROAD WORK ZONE LENGTH |  |
| Complies with AS 1742.3 |  |
| TRAFFIC SIGNALS, INTELLIGENT TRANSPORT SYSTEMS (ITS), VARIABLE SPEED LIMIT ZONES, AND TRAFFIC COUNT STATIONS |  |
| Traffic signals as specified |  |
| Variable speed limit zones as specified |  |
| Traffic count stations as specified |  |
| PORTABLE TRAFFIC SIGNALS |  |
| Complies with AS 4191 and AS 1742.3 |  |
| Contractor’s name and contact information on each portable traffic signal |  |
| Contact details of personnel available outside working hours |  |
| Site distance not less than 150 m |  |
| RESTORATION |  |
| Warning devices and traffic control measures removed |  |
| Temporary works and detours reinstated as specified |  |
| Areas impacted by the works stabilised |  |
| Complied with the requirements of Environmental Approvals and Clearances issued |  |
| Permanent traffic devices temporarily removed are reinstated |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – CLEARING, GRUBBING, AND REHABILITATION

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| TREES TO BE RETAINED |  |
| Trees to be retained clearly marked |  |
| Protection provided |  |
| Compensation for damage to trees marked to be retained or replace tree with equivalent size and species |  |
| MULCHING |  |
| Mulching is as specified |  |
| Stockpile heights 2 m maximum |  |
| Stockpile locations |  |
| STRIPPING OF TOP LAYER |  |
| Correct depth |  |
| Stockpile heights 2 m maximum |  |
| Stockpile locations |  |
| TREATMENT OF EXISTING SEALED SURFACES |  |
| As specified |  |
| SCARIFYING OF EXISTING ROADS |  |
| Scarify as specified |  |
| Existing seal removed |  |
| 200 mm cover minimum to buried material if burial permitted |  |
| GRUBBING |  |
| Correct depth removed |  |
| Grub holes filled and compacted |  |
| REINSTATEMENT |  |
| As specified |  |
| Mulch provided as specified |  |
| Detours rehabilitated as specified |  |
| CLEANING UP |  |
| Site is free of excess fill, rubble and other debris |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – EARTHWORKS

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

Give sufficient notice so that the Superintendent may:

| Spec. Ref. | Item | Checked – Initial & date |
| --- | --- | --- |
| #.16.3 | Witness proof rolling |  |
| #.16.4 | Approve subgrade conformance |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| GRVEL |  |
| From Pastoral Leasehold land at no cost to Principal |  |
| EARTHWORKS IN CUT |  |
| Excess material dumped and spread as specified |  |
| Rock and loose material removed as specified |  |
| Unsuitable material below subgrade surface other than rock removed as specified |  |
| EARTHWORKS IN FILL |  |
| Compaction prior to filling is complete |  |
| Benching as specified |  |
| Unsuitable material beneath fill has been replaced |  |
| Correct construction method is as specified |  |
| FILL MATERIAL |  |
| Fill type is as specified |  |
| PREPARATION AND MAINTENANCE OF SUBGRADE SURFACE |  |
| Surface is free draining and free of depressions |  |
| Tolerances as specified |  |
| EARTHWORKS FOR DRAINAGE |  |
| Stream diversions provided as shown on the drawings |  |
| Levees constructed to the locations, and to the dimensions shown on the drawings |  |
| Table drains constructed to the dimensions shown on the drawings |  |
| Table drains graded to prevent water ponding |  |
| Table drain offlets constructed at specified intervals |  |
| Table drain offlets constructed as specified |  |
| Table drain offlets graded to drain water without damaging surface |  |
| Table drain blocks constructed as and when specified |  |
| Catch drains constructed as and when specified |  |
| WIDENING OF EXISTING FORMATION |  |
| Constructed as shown on the drawings and as specified |  |
| TRIM AND COMPACT UNPAVED AREAS |  |
| Test as specified in Table – Test Frequencies for Soils – Part 3 of 3 in CONFORMANCE TESTING |  |
| SURFACE FORMATION |  |
| Constructed as shown on the drawings and as specified |  |
| BATTER PROTECTION BY GRASSING |  |
| Grassing as specified |  |
| Seed mixture and fertiliser as specified |  |
| Grass seed application as specified |  |
| Establishment and coverage as specified |  |
| BRIDGE FOUNDATIONS |  |
| Excavation is to required lines and levels |  |
| Excess material disposed of as specified |  |
| Foundations cut to a firm surface, with loose material removed |  |
| COFFER-DAMS SHORING AND SHEETING |  |
| Adequate height and depth |  |
| BACKFILLING |  |
| Excavation backfilled up to natural surface level using correct material and compacted to specified density |  |
| FILL ADJACENT TO BRIDGE STRUCTURES |  |
| Fill as specified |  |
| ABUTMENTS AND WINGWALLS |  |
| Fill compacted as specified |  |
| Weep holes and drainpipes provided as specified |  |
| COMPACTION |  |
| Is free of cracking and compaction planes |  |
| CONFORMANCE |  |
| Earthworks finished to a smooth compacted and uniform surface |  |
| Works meet tolerances as specified |  |
| PROOF ROLLING |  |
| Plant as specified |  |
| Level checks as specified |  |
| Compliance as specified |  |
| CONFORMANCE TESTING |  |
| As specified |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – CONFORMANCE TESTING

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| PANEL PERIOD CONTRACTORS |  |
| List supplied to Contractor |  |
| ORDERING TESTING |  |
| Tests ordered in writing |  |
| Correct rotation of Panel Period Contractors |  |
| TABLES - TEST FREQUENCIES, COMPLIANCE TESTING |  |
| Correct test frequencies |  |
| Reporting and completion times complied with |  |
| CONFORMANCE TESTING RESULTS |  |
| NATA endorsed test results provided within the specified time frame |  |
| LOT TESTING GENERALLY |  |
| Lots numbered with logical system |  |
| Lots selected by specified criteria |  |
| Lots subjected to conformance testing as specified |  |
| CONFORMANCE OF COMPACTION FOR SOILS |  |
| Dry density test results achieved |  |
| Test excavations backfilled with cement stabilised material |  |
| CONFORMANCE OF COMPACTION FOR ASPHALT |  |
| Relative compaction calculated as per specified formula |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – PAVEMENTS AND SHOULDERS

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

Notice for inpections/witnessing

Give sufficient notice so that the Superintendent may witness:

| Spec. Ref. | Item | Checked – Initial & date |
| --- | --- | --- |
| #.7.2 | Proof rolling |  |
| #.7.5 | Final pavement layer integrity |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| MATERIAL PROPERTIES |  |
| Gravel as specified |  |
| Fine crushed rock as specified |  |
| Blends of gravel and crushed rock as specified |  |
| Sand clay as specified |  |
| CONSTRUCTION OF PAVEMENT LAYERS |  |
| Mixing and placing as specified |  |
| Compaction as specified |  |
| Dry density ratios conform to Table - Dry Density Ratios for Conformance in the CONFORMANCE TESTING worksection |  |
| RECONSTRUCTION AND REHABILITATION OF EXISTING PAVEMENTS |  |
| Widening as specified |  |
| Strengthening by granular overlay on existing pavement as specified |  |
| Strengthening by granular overlay on re-worked existing sealed pavement as specified |  |
| SUPPLY TO STOCKPILE |  |
| Site clear and free draining |  |
| Sub-base gravel as specified |  |
| Stockpile shape as specified |  |
| PAVEMENT ACCEPTANCE |  |
| Pavement and shoulders acceptance as specified |  |
| Proof Rolling Requirement: - Proof rolling procedure - Plant conformance - Proof rolling compliance |  |
| Conformance testing requirements: - Ordering procedures as specified - Test holes backfilled |  |
| Surface roughness IRI less than 2.4 |  |
| Final pavement layer integrity requirement as specified |  |
| Dry back requirement as specified |  |
| Other tolerance requirements as specified |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – STABILISATION AND MODIFICATION

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| MATERIALS |  |
| As specified |  |
| IN SITU STABILISATION |  |
| Preliminary trial complete |  |
| Preparation of layer as specified |  |
| Spread rate as specified |  |
| Mixing plant as specified |  |
| Cement stabilisation as specified |  |
| Lime stabilisation as specified |  |
| Compaction is complete |  |
| Final surface is as specified |  |
| Any non-complying areas reconstructed |  |
| Longitudinal joints installed as and where specified |  |
| Transverse joints installed as and where specified |  |
| PLANT MIX STABILISATION |  |
| Preliminary trial provided where specified |  |
| Mixing plant is correct |  |
| Material is uniform and without segregation |  |
| Laying as specified |  |
| Paving machine as specified |  |
| Grader laying, permitted and appropriate |  |
| CONFORMANCE |  |
| Tolerances for stabilised layers are as specified |  |
| Stabiliser application rate is correct |  |
| Stabiliser content is correct |  |
| CONFORMANCE TESTING |  |
| As specified |  |
| Test holes backfilled |  |
| SURFACE ROUGHNESS REQUIREMENT |  |
| As specified IRI less than 2.4 |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – SPRAY SEALING

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

NOTICE FOR INSPECTIONS/WITNESSING

Give sufficient notice so that the Superintendent or nominated person may witness process:

| Spec. Ref. | Item | Checked – Initial & date |
| --- | --- | --- |
| #.9.5 | Collection of binder samples |  |
| #.13 | Spraying |  |
| #.13.4 | Preparation for sprayer run |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| MATERIAL REQUIREMENTS |  |
| Aggregates conform to required grade and properties |  |
| Cutter properties as specified |  |
| Pre-coat and adhesion agents as specified |  |
| Bitumen as specified |  |
| Cutback bitumen as specified |  |
| Bitumen emulsion as specified |  |
| Polymer modified binder properties as specified |  |
| Geofabric properties as specified |  |
| SPRAYERS AND PERSONNEL |  |
| Sprayers AAPA accredited |  |
| Sprayers calibrated |  |
| Plant and equipment must be in good working condition |  |
| FINAL PREPARATION OF PAVEMENT SURFACE |  |
| Raised reflective pavement markers removed |  |
| Loose stones, dust and foreign matter removed |  |
| Surface dampened prior to spraying |  |
| Water removed from surface |  |
| SETTING OUT |  |
| Set out by a qualified surveyor |  |
| BINDER COAT REQUIREMENTS |  |
| As specified |  |
| PRIME PRIMER SEALS AND ENRICHMENT COATS |  |
| As specified |  |
| Correct temperature prior to spraying |  |
| STRAIGHT RUN BINDER COATS |  |
| Approval from the Executive Director Civil Services required |  |
| Material is as specified |  |
| POLYMER MODIFIED BINDER COATS |  |
| Material as specified |  |
| SAMPLING OF BINDER |  |
| Test requests required |  |
| Sampling containers 1 litre minimum, clean, and leak proof |  |
| Frequency of samples to CONFORMANCE TESTING worksection |  |
| Collection of samples as specified |  |
| Sample identification as specified |  |
| STOCKPILE SITES |  |
| Not permitted in urban areas |  |
| 15 m between adjacent different aggregate size sites |  |
| Sites well drained and on hard ground |  |
| Access roads maintained |  |
| Stockpiles covered |  |
| PRECOATING AGGREGATE |  |
| As specified |  |
| ADHESION AGENT |  |
| As specified |  |
| SPRAYING |  |
| Correct atmospheric conditions for spraying |  |
| Sprayer preparation as specified |  |
| Spray bar clean and aligned correctly |  |
| Spray bar at correct height |  |
| Application spray rates as specified |  |
| Protective paper strips used at start and finish of each run |  |
| Installation of temporary pavement markers as specified |  |
| Sprayer run as specified |  |
| APPLICATION OF GEOFABRIC |  |
| As specified |  |
| APPLICATION OF AGGREGATE |  |
| Aggregate is clean and ready for application |  |
| Aggregate spread even and uniformly over primed surface |  |
| Rolling rate as specified |  |
| WASTE MATERIAL |  |
| Waste material removed from site |  |
| CONFORMANCE |  |
| Final surface as specified |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – DENSE GRADED ASPHALT

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| TRAFFIC CATEGORIES |  |
| Mix types and binder types used appropriate to relevant traffic categories |  |
| MATERIALS |  |
| Course aggregates as specified |  |
| Fine aggregates as specified |  |
| Mineral filler as specified |  |
| Bituminous binder as specified |  |
| Bitumen emulsion conforming to AS 1160 |  |
| Additives approved |  |
| Reclaimed asphalt pavement as specified |  |
| ASPHALT MIX DESIGN |  |
| Mix type as specified |  |
| Design mix registered |  |
| SURFACE PREPARATION |  |
| Surface swept clean |  |
| Foreign matter removed |  |
| Surface prepared as specified |  |
| Free from laminations |  |
| Depressions filled with an asphalt correction course layer |  |
| Bituminous spray seal prime coat applied |  |
| Resurfacing of existing bitumen and concrete surfaces as specified |  |
| Cold planning as specified |  |
| Shape and surface roughness as specified |  |
| Tack coat applied |  |
| MIXING |  |
| Job mix as specified |  |
| Correct temperature as specified |  |
| Storage not more than 24 hours |  |
| TRANSPORT AND SUPPLY |  |
| Transport vehicles as specified |  |
| Asphalt material covered |  |
| Rate of supply as specified |  |
| SPREADING |  |
| Minimum laying temperatures as specified |  |
| Mechanical spreading as specified |  |
| Laying pattern as specified |  |
| Joints installed as specified |  |
| Temporary ramps as specified |  |
| COMPACTION |  |
| Plant and equipment free of oil and fuel leaks |  |
| Correct methodology used |  |
| Roller type used as specified |  |
| Initial rolling complete |  |
| Intermediate rolling as specified |  |
| Final rolling as specified |  |
| Joint compaction is as specified |  |
| Tamping as specified |  |
| CONFORMANCE |  |
| Conformance testing as specified |  |
| Process testing reports |  |
| Finished pavement properties as specified |  |
| Conformance of asphalt production as specified |  |
| Conformance sampling and testing frequencies as specified |  |
| Asphalt compaction as specified |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – SLURRY SURFACING

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

TESTS

| Spec. Ref. | Item | Checked – Initial & date |
| --- | --- | --- |
| #.10 | Conformance testing – random – requested by Superintendent |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| TRAFFIC CONTROL |  |
| Traffic control requirements as specified. |  |
| MATERIALS |  |
| Binder material as specified |  |
| Aggregates as specified |  |
| Water potable and free from impurities |  |
| Mineral filler as specified |  |
| Samples |  |
| Stockpiles as specified |  |
| PLANT AND EQUIPMENT |  |
| Mixing machine as specified |  |
| Spreader box distributor as specified |  |
| Ancillary plant as specified |  |
| MIX REQUIREMENTS |  |
| Mix requirements as specified |  |
| Design mixes test reports submitted to Superintendent |  |
| PREPARATION AND SETOUT |  |
| Pavement swept clean |  |
| Previously sealed areas prepared as specified |  |
| Foreign material removed |  |
| Set out as specified |  |
| APPLICATION |  |
| No application during rain or if rain is expected |  |
| Cleared build up from slurry box |  |
| End of runs square and feathered |  |
| Minimal longitudinal joints |  |
| Damped pavement if ambient temperature over 40°C |  |
| CONFORMANCE TESTING |  |
| Sampling of the mixed material as specified |  |
| Testing required after identification of excess aggregate loss |  |
| SURPLUS WASTE AND DEFECTIVE MATERIAL |  |
| Surplus material removed from site |  |
| Waste is correctly disposed of |  |
| MAINTENANCE |  |
| Keep the completed work in a satisfactory condition for 1 month after completion |  |
| Maintenance will be limited to failures as a result of the operations of the contractor |  |
| RECORDS |  |
| Slurry Surfacing Daily Record Sheet – Spray Surfacing |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – MISCELLANEOUS CONCRETE WORKS

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

NOTICE FOR INPECTIONs/WITNESSING

Give sufficient notice so that the Superintendent may inspect/witness:

| Spec. Ref. | Item | Checked – Initial & date |
| --- | --- | --- |
| #.6.2 | Formwork inspection before concrete being placed |  |
| #.6.3 | Placing of concrete witnessing |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| MATERIALS |  |
| Manufacturer's test certificates required for cement, aggregate and reinforcement |  |
| Cement as specified |  |
| Fine aggregate as specified |  |
| Coarse aggregate as specified |  |
| Water as specified |  |
| Chemical admixtures as specified |  |
| Reinforcement as specified |  |
| Recycled crushed glass as specified |  |
| CONCRETE |  |
| Ready mix concrete as specified |  |
| Job mixed concrete as specified |  |
| Addition of admixtures as specified |  |
| FOUNDATIONS |  |
| Provided as specified |  |
| CONSTRUCTION |  |
| Formwork installed inspected before placing concrete |  |
| Placing of concrete as specified |  |
| Construction joints installed as specified |  |
| Expansion/contraction joints installed as specified |  |
| Tooled joints installed as specified |  |
| Finished surface is smooth and even in colour |  |
| Exposed edges rounded off |  |
| Exposed surfaces protected until fully cured |  |
| Exposed aggregate finish smooth |  |
| Backfilling material as specified |  |
| Backfill material compacted |  |
| CONFORMANCE |  |
| Tolerances as specified |  |
| DEFECTIVE CONCRETE AND MATERIALS |  |
| Defective concrete removed as directed by the Superintendent |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – DRAINAGE WORKS

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| CLEARING |  |
| Site cleared |  |
| MATERIALS |  |
| Pipes and box culverts marked with a Standards Australia conformance stamp |  |
| Pre cast reinforced concrete pipes: - Correct type - Class is as specified |  |
| Rubber ring joint pipes class clearly marked |  |
| Precast reinforced concrete box culverts supplied as specified |  |
| Bedding material as specified |  |
| Concrete as specified |  |
| Mortar as specified |  |
| Select fill as specified |  |
| CONSTRUCTION OF CULVERTS AND STRUCTURES |  |
| Setting out as specified |  |
| Excavation as specified |  |
| Bedding as specified |  |
| Cover for construction loads |  |
| Laying as specified |  |
| Backfill as specified |  |
| INLET AND OUTLET STRUCTURES, PITS, HEADWALLS, AND OTHER STRUCTURES |  |
| As specified |  |
| INLET AND OUTLET CHANNELS |  |
| As specified |  |
| OPEN UNLINED DRAINS |  |
| Drains trimmed to form neat levees |  |
| Compaction as specified |  |
| Allowance for natural surface run-off provided |  |
| REMOVAL OF EXISTING CULVERTS AND DRAINAGE STRUCTURES |  |
| Demolished and removed |  |
| SUBSOIL DRAINS |  |
| Excavation as specified |  |
| Filter materials as specified |  |
| Geotextile fabric supplied and installed as specified |  |
| Subsoil drain pipe supplied and installed as specified |  |
| Pipe laying and backfilling is as specified |  |
| End walls constructed as specified and marked with guide posts |  |
| Galvanised wire mesh installed at opening |  |
| CONFORMANCE |  |
| Tolerances conform to Table – Drainage Works Tolerances |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – PROTECTION WORKS

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| FOUNDATIONS |  |
| As specified |  |
| GEOTEXTILE FABRICS |  |
| As specified |  |
| Storage, packaging and handling as specified |  |
| Certificate of Compliance received |  |
| Construction as specified |  |
| Grades as specified |  |
| Conformance testing as specified |  |
| ROCK PROPERTIES |  |
| As specified |  |
| STONE PITCHING |  |
| Stone material as specified |  |
| Stones firmly bedded in layers |  |
| Grouted stone pitching: - Mortar type as specified - Weep holes as specified |  |
| DUMPED ROCK PROTECTION |  |
| Rock dimensions as specified |  |
| Rocks dumped in specified area |  |
| Adjacent areas protected from damage |  |
| Construction within tolerance |  |
| QUARTER TONNE CLASS DUMPED ROCK PROTECTION |  |
| Boulder dimensions as specified |  |
| Boulders dumped in specified area |  |
| Adjacent areas protected from damage |  |
| Construction within tolerance |  |
| RUBBLE |  |
| Material as specified |  |
| Material dumped in specified area without segregation |  |
| Material compacted to produce a tight even finish |  |
| Construction within tolerance |  |
| GABIONS |  |
| Steel wire mesh as specified |  |
| Tensile strength as specified |  |
| Galvanizing as specified |  |
| PVC coating as specified |  |
| Mesh edge protection with selvedge wire as specified |  |
| Construction: - As per manufacturer’s instructions - Wire framework pretension as specified - Spreaders provided - Stone properties as specified - Mesh and wiring tightness as specified |  |
| RENO MATRESSES |  |
| Steel wire mesh as specified |  |
| Tensile strength as specified |  |
| Galvanizing as specified |  |
| PVC coating as specified |  |
| Mesh edge protection with selvedge wire as specified |  |
| Construction: - As per manufacturer’s instructions - Wire framework pretension as specified - Spreaders provided - Stone as specified - Mesh and wiring tightness as specified |  |
| REVETMENT MATTRESSES |  |
| Toe-in provided as specified |  |
| Mortar mix proportions as specified |  |
| Stitching and seams as specified |  |
| Mattress anchored before pumping concrete |  |
| Openings provided |  |
| Openings sealed on completion of mortar pumping |  |
| EMBANKMENT PROTECTION - CONCRETE |  |
| Construction as specified |  |
| Drainage holes provided as specified |  |
| MARGINS |  |
| Construction joints installed as specified |  |
| Finish is flush with top of pavement |  |
| MARGINS TOLERANCES |  |
| As specified |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – ROAD FURNITURE AND TRAFFIC CONTROL DEVICES

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| TACTILE GROUND SURFACE INDICATORS |  |
| As specified conforming to AS 1428.4.1 |  |
| FENCING |  |
| Fence line cleared |  |
| Fence erected as specified |  |
| Owner/lessee permission to enter land |  |
| Gates provided where specified |  |
| Materials as specified |  |
| Stock fences to CS 3310 |  |
| Security fences to CS 3308 |  |
| Temporary site safety fence as specified |  |
| Pedestrian fences to CS 3307 |  |
| Log barrier fence as specified |  |
| Vehicle movement barriers/fences to CS 3305 |  |
| Cyclist holding rails to CS 3302 and CS 3305 |  |
| Recycled plastic bollards as specified |  |
| Culvert crossing guard rail as specified |  |
| FLEXIBLE GUIDE POSTS |  |
| Properties as specified |  |
| Installation to CS 3500 |  |
| Certification received |  |
| Characteristics as specified |  |
| Delineator properties as specified |  |
| Installation of delineators as specified |  |
| ROAD SIGNS |  |
| Anti-spear fixings for hazard markers (sight boards) as specified |  |
| Non reflective materials as specified |  |
| Reflective material as specified |  |
| Blanks material as specified |  |
| Manufacture stamp as specified |  |
| Post sizes as specified |  |
| Anti-graffiti coating as specified |  |
| Location and lateral placement as specified |  |
| Height as specified |  |
| Installation as specified |  |
| Reinstatement and relocation of existing signs as specified |  |
| FLOOD GAUGE POSTS |  |
| Posts and gauges to CS 3501 |  |
| Installation as specified |  |
| CATTLE GRIDS |  |
| Installed to CS 3310, CS 3313, CS 3314, and CS 3315, and as specified |  |
| ROAD SAFETY BARRIERS - STEEL RAIL |  |
| Materials are as specified |  |
| Marking of materials as specified |  |
| Certificates of compliance provided |  |
| Installation is as specified |  |
| ROAD SAFETY BARRIERS – STEEL WIRE ROPE SYSTEM |  |
| Materials and installation as specified |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – PAVEMENT MARKING

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

TESTS

Give sufficient notice so that the Superintendent may witness testing:

| Spec. Ref. | Item | Checked – Initial & date |
| --- | --- | --- |
| #.10 | Field testing |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| CONTRACTOR ACCREDITATION |  |
| All work must be carried out by those with the relevant PCCP accreditation |  |
| PAVEMENT MARKING |  |
| Paint colours correct |  |
| GLASS BEADS |  |
| Type as specified |  |
| PAVEMENT MARKING SETTING OUT |  |
| Set out to AS 1742 and Department standard drawings |  |
| PAVEMENT MARKING APPLICATION |  |
| Substrate is free from dirt, loose detritus, mud and other extraneous matter |  |
| Substrate is dry |  |
| Application as specified |  |
| Glass beads applied as specified |  |
| Dimensional tolerances as specified |  |
| Pavement markings protected from traffic |  |
| FIELD TESTING |  |
| Wet film thickness as specified |  |
| Glass bead application as specified |  |
| Wear assessment limits as specified |  |
| Degree of wear as specified (95% intact at 6 months) |  |
| THERMOPLASTIC MATERIALS |  |
| Thermoplastic road marking material as specified |  |
| COLD APPLIED PLASTIC MATERIALS |  |
| Material as specified |  |
| AUDIO TACTILE LINE MARKING (ATLM) |  |
| Material as specified |  |
| Tolerances as specified |  |
| Substrate is free from dirt, loose detritus, mud and other extraneous matter |  |
| Substrate is dry |  |
| Tack coat applied |  |
| Application as specified |  |
| Minimum reflectivity achieved |  |
| RAISED RETROREFLECTIVE PAVEMENT MARKERS (RRPMS) |  |
| Material as specified |  |
| Substrate is free from dirt, loose detritus, mud and other extraneous matter |  |
| Substrate is dry |  |
| Marker placement as specified |  |
| Reflectors face oncoming traffic |  |
| REMOVAL OF PAVEMENT MARKINGS |  |
| Removal as specified |  |
| No visible ghosting |  |
| Surface defects repaired after removal |  |
| RESURFACING CONTRACTS |  |
| Co-ordination |  |
| Removal of temporary pavement markers |  |
| Removal of temporary traffic control signage |  |
| Reporting |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – LANDSCAPE

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

TESTS

Give sufficient notice so that the Superintendent may witness testing:

| Spec. Ref. | Item | Checked – Initial & date |
| --- | --- | --- |
| #.7.7 | Hydroseeding soil testing |  |
| #.8.7 | Irrigation - Testing |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| MATERIALS |  |
| Trees, shrubs and groundcovers conform to specified characteristics |  |
| Grass seed conforms to specification requirements |  |
| Fertilisers supplied as specified |  |
| Imported soils conform to requirements |  |
| Insecticide used in accordance with manufacturer’s instructions |  |
| Mulch supplied as specified |  |
| SITE PREPARATION |  |
| Set out is in accordance with the drawings |  |
| Existing vegetation is protected as specified |  |
| Unwanted vegetation removed |  |
| Finished surface levels free draining |  |
| Topsoil supplied as specified |  |
| Topsoil stockpile free draining and not exceeding 2 m height |  |
| PLANTING |  |
| Set out as per drawings |  |
| Preparation and treatment of holes as specified |  |
| Plants supplied from approved nursery |  |
| Planting of trees, shrubs and ground cover as specified |  |
| Holes backfilled, dished at top to retain water |  |
| Watering, maintenance and mulching as specified |  |
| GRASSING |  |
| Ground preparation as specified |  |
| Seeds as specified |  |
| Application rates as specified |  |
| Fertilisers applied as specified |  |
| Reseeding as specified |  |
| Irrigation as specified |  |
| Establishment as specified |  |
| Mowing as specified |  |
| BATTER PROTECTION BY HYDROSEEDING |  |
| Material properties as specified |  |
| Materials delivered in UV and weather-resistant factory labelled packages |  |
| Storage as per manufacturers recommendations |  |
| Materials, composition, packaging , as specified |  |
| Soil analysis test results |  |
| Species selection as specified |  |
| Substrate preparation as specified |  |
| Installation as specified |  |
| Application as specification |  |
| Protection of treated areas in place |  |
| IRRIGATION |  |
| Irrigation system supplied as specified |  |
| Materials supplied as specified |  |
| Set out as per drawings |  |
| Excavation carried out as specified |  |
| Installation: - Approval from Power and Water Corporation before connecting to water supply system - Conduits and pipes grade or class identification marking visible for inspection - Backflow prevention device installed - 50mm compacted sand bedding provided - Pressure testing carried out - Backfilling carried out as per specification |  |
| TELEMETRIC CONTROL STATION DETAILS |  |
| Materials as specified: - Mounting posts - Stainless steel whip aerial - Irrigation controller - Radio - Solar panel |  |
| Installation as per drawings |  |
| Installation as specified |  |
| ESTABLISHMENT PERIOD |  |
| Site kept neat and tidy |  |
| Site is weed free |  |
| All plants and grass healthy |  |
| Damaged, dying or dead plants replaced within 14 days |  |
| All plantings free from insects, pests and diseases |  |
| Mulch maintained |  |
| Stakes and ties remain secured |  |
| Grass mowed when height exceeds 100 mm |  |
| Establishment period records |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – DUCTING AND CONDUITS

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| SUPPLY AND INSTALLATION |  |
| Termination of duct crossing as specified |  |
| Trench excavated as specified |  |
| Minimum cover achieved |  |
| Sand bedding in place |  |
| Marker tape installed |  |
| Backfill material as specified |  |
| MARKINGS |  |
| Kerbed areas marked to identify ducts |  |
| Star pickets and identification plate provided |  |
| CONDUIT MARKERS |  |
| Installed as specified |  |
| ELECTRICAL DUCTING |  |
| Correct type |  |
| Earth strap installed |  |
| Marker tape installed |  |
| Installation done in liaison with Power and Water Corporation |  |
| WATER DUCTING |  |
| Correct type |  |
| Installation done in liaison with Power and Water Corporation |  |
| TELSTRA DUCTING |  |
| Liaison with Telstra |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – TRAFFIC CONTROL SIGNALS AND INTELLIGENT TRANSPORT SYSTEMS

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

TESTS

Give sufficient notice so that the Superintendent may witness testing:

| Spec. Ref. | Item | Checked – Initial & date |
| --- | --- | --- |
| #.10 | Checking and testing – signal controller(s), cabling, and lanterns |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| GENERAL EQUIPMENT AND INSTALLATION |  |
| Surge reduction filter installed |  |
| Posts and fittings are galvanised |  |
| Continuous welds |  |
| Footing installed as specified |  |
| Footways reinstated |  |
| Concrete support collar installed |  |
| Vehicle signal lanterns: - Correct positioning - Target board installed - Temporary covers provided - Inspection hole provided to covers |  |
| Pedestrian signal lanterns: - Correct type - Correct positioning |  |
| Traffic signal controller: - Location as specified - Foundation block installed - Earth stake installed |  |
| Finish colours as specified |  |
| TRAFFIC SIGNAL CABLING |  |
| Multicore connecting cable: - 51 core multicore to AS/NZS 2276.1 |  |
| Loop detector feeder cables: - Correct type to AS/NZS 2276.2 - Installed as specified |  |
| Loop cable for vehicle detectors: - Correct type to AS/NZS 2276.3 - Saw cut in pavement cleaned out - Cable installed - Backfill as specified |  |
| Power supply cable: - Correct type - Connection as specified |  |
| Cable conduit: - Draw wire provided - Installed as specified |  |
| Conduit junction pit: - Installed as specified - Coarse gravel bed in place - Lids are level with adjacent surface |  |
| Detector pits: - Installed as specified - Sand bed in place |  |
| PEDESTRIAN PUSH BUTTON ASSEMBLIES |  |
| Units installed as specified |  |
| AUDIO-TACTILE TRAFFIC SIGNAL EQUIPMENT |  |
| Provided as specified |  |
| SCATS FACILITIES |  |
| Communications isolation pillar installed as specified |  |
| Connections between controller and communications isolation pillar complete |  |
| Connections to Telstra pit from communications isolation pillar complete |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – TRAFFIC COUNTING STATIONS

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

TESTS

Give sufficient notice so that the Superintendent may witness testing:

| Spec. Ref. | Item | Checked – Initial & date |
| --- | --- | --- |
| #.10 | Testing of installation |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| CABINETS |  |
| Provided by Principal |  |
| Installed as specified |  |
| Terminal blocks provided as specified |  |
| Drawings received |  |
| Switchboard for Solar power site installed as specified |  |
| CABLING |  |
| Provided as specified |  |
| Insulation as specified |  |
| CONDUIT |  |
| Correct type as specified |  |
| Marker tape provided |  |
| Termination as specified |  |
| Minimum cover 450 mm as specified |  |
| POLE ASSEMBLEY |  |
| Provided by Principal |  |
| Opening provided in solar panel backing plate |  |
| INSTALLATION |  |
| Excavation as specified |  |
| Backfilling as specified |  |
| Concrete foundation for pole assembly installed as specified |  |
| Detector pits provided as specified |  |
| INSTALLATION – VEHICLE DETECTION |  |
| Detector loop cable type correct |  |
| Detector loop saw cut as specified |  |
| Detector loop saw cut backfilled as specified |  |
| Piezo axle sensors and guides supplied by Principal |  |
| Piezo axle sensor guide installation as specified |  |
| Piezo axle sensor installation as specified |  |
| SOLAR POWER SUPPLY |  |
| Solar panel secured to mounting plate |  |
| Wiring provided as specified |  |
| Solar panel facing North as specified |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – STREET LIGHTING

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

TESTS

Give sufficient notice so that the Superintendent may witness testing:

| Spec. Ref. | Item | Checked – Initial and date |
| --- | --- | --- |
| #.11 | Testing and commissioning |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| MATERIALS |  |
| Comply with Power and Water Corporation Street Lighting manual |  |
| Comply with Power and Water Corporation standard drawings |  |
| Components suitable for terrain category |  |
| Street light luminaires type as specified on the drawings |  |
| Control equipment as specified |  |
| EXCAVATION |  |
| Trench width is as specified |  |
| Trench depth is as specified |  |
| Clearances to existing services as specified |  |
| FOOTINGS |  |
| Installed as specified |  |
| BACKFILLING |  |
| Material as specified |  |
| Cable installation as specified |  |
| Backfilling is as specified |  |
| Cable marker tapes provided as specified |  |
| INSTALLATION OF LIGHT COLUMNS |  |
| As per Power and Water Corporation standard drawings |  |
| CONNECTION |  |
| Service cables connected |  |
| Liaise with Power and Water Corporation – Connection to power network |  |
| EXISTING STREET LIGHTING |  |
| Dismantled and removed as specified |  |
| Salvaged items stored as specified |  |
| Existing concrete footings removed |  |
| Temporary lighting as specified |  |
| TESTING AND COMMISSIONING |  |
| Testing is complete |  |
| Power and Water Corporation contacted to energise new works |  |
| REINSTATEMENT |  |
| Reinstate any damaged items to their original condition |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – DIRECTIONAL BORING

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

TESTS

| Spec. Ref. | Item | Checked – Initial and date |
| --- | --- | --- |
| #.3 | Telstra |  |
|  | NBN Corporation |  |
|  | Power and Water Corporation |  |
|  | Jacana Energy |  |
|  | Territory Generation |  |
|  | Any gas supply agency or authority |  |
|  | Any data service provider |  |
|  | Any other Agency or Authority |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| STANDARDS, PUBLICATIONS, AND AUTHORITIES |  |
| Pipes and fittings to comply with AS 1579 |  |
| Compliance with utility authorities requirements |  |
| DIRECTIONAL BORING WITH PIPE CASTING |  |
| Jacking pit dimensions as small as reasonably possible |  |
| Pipe jacking equipment certified as fit for use |  |
| Casing as specified |  |
| Casing inside diameter is as specified |  |
| DIRECTIONAL BORING WITHOUT PIPE CASING |  |
| Bore hole as specified |  |
| Plastic skids used |  |
| Joints not stressed or separated |  |
| TESTING OF SERVICES |  |
| Testing of services to the applicable authority test requirements (see table above) |  |
| FILLING OF CAVITIES |  |
| Pressure service pipes: - Grout as specified - Pipe full of water at correct pressure |  |
| Non-pressure service pipes: - Bedding is as specified - Bedding thoroughly watered in |  |
| END CAPS |  |
| Fitted as specified |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |

DEPARTMENT OF INFRASTRUCTURE planning and logistics

# CHECK LIST – PROTECTIVE COATINGS

| PROJECT |  | SPEC. NO. |  |
| --- | --- | --- | --- |
| Checked by |  | Title |  |
| Work lot |  | Date |  |

TESTS

Give sufficient notice so that the Superintendent may witness testing:

| Spec. Ref. | Item | Checked – Initial & date |
| --- | --- | --- |
| #.9.2 | Equipment – Oil carry-over tests |  |

COMPLIANCE CHECKS – Refer Specification

| Item | Checked - Initial & Date |
| --- | --- |
| PROTECTIVE COATINGS |  |
| Surface preparation as specified |  |
| Prime coat as specified |  |
| PROTECTIVE COATING SPECIFICATIONS - SYSTEMS AND APPROVED PRODUCTS |  |
| Conforms to Tables - Protective coating specifications - Systems and approved products |  |
| CONTRACTOR RESPONSIBILITIES |  |
| Applicators PCCP accredited |  |
| Details of plant and equipment |  |
| Written details of experience |  |
| ITPs |  |
| Environmental Policy |  |
| Warranties |  |
| Pre job meeting – review specification and ITPs |  |
| Standard of workmanship to manufacturer’s instructions |  |
| SAFETY |  |
| Safety precautions for abrasive blasting and protective coatings application |  |
| Storage of hazardous materials |  |
| Safety Data Sheets (SDS) |  |
| TRAFFIC MANAGEMENT PLAN |  |
| Permit to Work in a Road Reserve |  |
| Traffic Management Plan |  |
| BARRIERS |  |
| Barriers and warning signs installed |  |
| Adjacent areas and equipment protected |  |
| Masking materials removed upon completion |  |
| EQUIPMENT |  |
| Conforming to the appropriate statutory Acts, Regulations and By-laws |  |
| Maintained as specified |  |
| Used as specified |  |
| ENVIRONMENTAL CONDITIONS |  |
| Environmental Test Reports AS 3894 parts 10, 11, and 12 |  |
| Reports to AS 3894 Parts 13 and 14 for structural steel |  |
| SURFACE PREPARATION |  |
| Defects removed |  |
| Contaminants removed |  |
| All welds smooth and merge evenly with joining surfaces |  |
| Abrasive blasting as specified |  |
| Spot and whip abrasive blasting as specified |  |
| Water washing and jetting as specified |  |
| APPLICATION OF PROTECTIVE COATINGS |  |
| Atmospheric conditions daily records |  |
| Coating material labels intact and seals unbroken |  |
| Material pot life valid |  |
| Application as specified |  |
| Materials as specified |  |
| Coatings free of sagging, pinholes, dry overspray and other defects |  |
| QA qualifications and documentation as specified |  |
| CONTRACTOR RECORDS |  |
| Daily reports |  |
| Wet film and dry film thickness |  |
| Appointment of inspector |  |
| HANDLING OF FINISH COATED ITEMS |  |
| As specified |  |

Action Required - ITC issued on items that do not comply

| Reported to: | Date: |
| --- | --- |

Final Check

| Made by: |  | Date: |
| --- | --- | --- |
| Comments |  |  |