|  | Questions are followed by answer fields. Use the ‘Tab’ key to navigate through. Replace Y/N or Yes/No fields with your answer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GD-CS-016B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This application should be read in conjunction with the departments GD-CS-016 Street Lighting Guideline. Completed applications should be sent to assetinformation.dli@nt.gov.au Note: This datasheet is for DLI only. The vendor is to fill the blank sections in the datasheet. The values shown are according to the current standards AS/NZS 1158. The should vendor state how many spare luminaires and parts can be stored. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vendor Details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vendor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vendor Contact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Email |  |  |  |  |  |  |  |  |  | Phone |  |  |  |  |  |  |  |
| Luminaire Datasheet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Luminaire make |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Luminaire model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Luminaire weight (kg) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LED chip make |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LED chip model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DALI dimmable drive make |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DALI dimmable drive model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Driver configuration DC output current supplied directly to each LED group (mA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Luminaire output (lumens) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Luminaire input power (watts) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total luminaire input power (watts) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maintenance factor (Must be ≤ 0.8) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Checklist |  |  |  |  | Yes | No |  | Comments |  |  |  |  |  |  |  |  |  |
| Integral tilt range -10 to 0 |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| National Electrical Manufacturers Association (NEMA) 7-pin base photoelectric (PE) cell |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| NEMA socket (ANSC136.41) Receptable (on top) |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| Required documents attached |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| Perfect lite printouts |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| Agi32 printouts |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| Agi32 *.AGI file attached |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| LM-80-08 full report attached |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| LM-79-08 full report |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| IES and CIE electronic files |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| ISTMT report |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| TM-21-11 calculations |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| RFP MF calculations |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| Complying Wieland plug and lead of correct length |  |  |  |  | ☐ | ☐ |  |  |  |  |  |  |  |  |  |  |  |
| Values of light technical parameters for Roads in local areas |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average horizontal illuminance (lx) Depending on the subcategory p |  |  |  |  | PR1 |  | PR2 |  |  |  |  | PR3 |  | PR4 |  |  | PR5 |
|  |  |  |  |  | 7 |  | 3.5 |  |  |  |  | 1.75 |  | 1.3 |  |  | 0.85 |
| Point horizontal illuminance (lx) |  |  |  |  | PR1 |  | PR2 |  |  |  |  | PR3 |  | PR4 |  |  | PR5 |
|  |  |  |  |  | 2 |  | 0.7 |  |  |  |  | 0.3 |  | 0.22 |  |  | 0.14 |
| Illuminance (horizontal) uniformity |  |  |  |  | PR1 |  | PR2 |  |  |  |  | PR3 |  | PR4 |  |  | PR5 |
|  |  |  |  |  | 8 |  | 8 |  |  |  |  | 8 |  | 8 |  |  | 10 |
| Values of light technical parameters for pathways and cyclist paths |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Average horizontal illuminance (lx) Depending on the subcategory p |  |  |  |  | PP1 |  | PP2 |  |  |  |  | PP3 |  | PP4 |  |  | PP5 |
|  |  |  |  |  | 10 |  | 7 |  |  |  |  | 3 |  | 1.5 |  |  | 0.8 |
| Point horizontal illuminance (lx) |  |  |  |  | PP1 |  | PP2 |  |  |  |  | PP3 |  | PP4 |  |  | PP5 |
|  |  |  |  |  | 2 |  | 1 |  |  |  |  | 0.5 |  | 0.25 |  |  | 0.14 |
| Illuminance (horizontal) uniformity |  |  |  |  | PP1 |  | PP2 |  |  |  |  | PP3 |  | PP4 |  |  | PP5 |
|  |  |  |  |  | 5 |  | 5 |  |  |  |  | 5 |  | 5 |  |  | 5 |
| Limitation of Luminous Intensity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Luminaire luminous flux |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ≤ 4000 lm |  |  |  |  |  |  |  |  | > 4000 lm |  |  |  |  |  |  |  |  |
| 720 cd absolute |  |  |  |  |  |  |  |  | 180 cd/1000 lm |  |  |  |  |  |  |  |  |
| 300 cd absolute |  |  |  |  |  |  |  |  | 80 cd/1000 lm |  |  |  |  |  |  |  |  |
| Upward waste light ratio |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Luminaire application |  |  |  |  |  |  |  |  | Maximum UWLR light source type |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Non-SSL |  |  |  |  |  |  | SSL |  |
| Local roads (PR, PP and PC subcategories) |  |  |  |  |  |  |  |  | 4% |  |  |  |  |  |  | 1% |  |
| Public spaces (PA and PE subcategories) |  |  |  |  |  |  |  |  | 5% |  |  |  |  |  |  | 3% |  |
| Authorised Officer Details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Authorised Officer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Email |  |  |  |  |  |  |  |  |  | Phone |  |  |  |  |  |  |  |
| Signature |  |  |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |  |
| Vendor Signature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vendor Contact |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Signature |  |  |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |  |
| End of form |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| End of form |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Internal use only - Assessing officer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Recommended or not recommended |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  |  |  |  |  |  |  |  | Position |  |  |  |  |  |  |
| Signature |  |  |  |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |
| Delegate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Approved or not approved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Signature |  |  |  |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |
| Organisation advised |  |  |  | Yes / No |  |  |  |  |  |  | Date |  |  |  |  |  |  |
| End of form |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |