# Project Specific Requirements

DLI – SSCM v9.2b – PSRs – v.4 – 20 July 2026

Reference

Read these Project Specific Requirements in conjunction with the Standard Specification for Civil Maintenance (SSCM) and the Drawings if any. Only those parts of the Standard Specification which refer to the works being carried out apply.

Standard Specification for Civil Maintenance – Reference Text

The technical specification for this project is the Standard Specification for Civil Maintenance v9.2b which is a separate printed document of reference text.

Site Copy

Retain a current copy of the Standard Specification for Civil Maintenance on site for the duration of the Contract. Retain a copy of these Project Specific Requirements on site for the duration of contract.

Electronic copies are available: https://dipl.nt.gov.au/industry/technical-standards-guidelines-and-specifications/technical-specifications/roads

Project Specific Requirements

The selection of specific items or materials for the works being carried out are as specified in these Project Specific Requirements or shown as notes on the drawings.

Clause Numbering

The clause numbers in the Project Specific Requirements remain consistent and refer back to the relevant clause and section in the Standard Specification for Civil Maintenance.

Precedence

Any provision in these Project Specific Requirements or on the project drawings shall override any conflicting provision in the Standard Specification.

Contents

SSCM Section 1	Miscellaneous Provisions

SSCM Section 2	Provision for Traffic

SSCM Section 3	Earthworks and Drainage

SSCM Section 4	Conformance Testing

SSCM Section 5	Grading and Gravel Sheeting

SSCM Section 6	Stabilisation Maintenance

SSCM Section 7	Spray Sealing Maintenance

SSCM Section 8	Bituminous Surface Maintenance

SSCM Section 9	Concrete Maintenance

SSCM Section 10	Drainage Maintenance

SSCM Section 11	Protection Works Maintenance

SSCM Section 12	Road Furniture Maintenance

SSCM Section 13	Pavement Marking Maintenance

SSCM Section 14	Landscape Maintenance

SSCM Section 15	Slashing and Weed Control

SSCM Section 16	Traffic Signals and Intelligent Transport Systems Maintenance

SSCM Section 17	Street Lighting Maintenance

SSCM Section 18	Street Sweeping

SSCM Section 19	Aerodrome and Aeroplane Landing Area Maintenance

SSCM Section 20	Bus Stop Maintenance

SSCM Section 21	Road and Marine Amenity Maintenance

SSCM Section 22	Protective Coatings

SSCM Section 23	Measurement and Payment

SSCM Section 24	Referenced Australian Standards

SSCM Section 25	Other Referenced Authorities and Documents

SSCM Section 26	Acts, Regulations and Codes

SSCM Section 27	Civil Standard Drawings for Road Maintenance

SSCM Section 28	Northern Territory Climate Zones Table

Edit the Project Specific Requirements to include only those sections required for the project. Do not change any section or clause numbers as they refer directly to the section and clause numbers in the standard specification.

If any clauses are amended by these PSRs, show the clause number in its numbering order place; State the requirement and state that the clause in the PSR overrides the equivalent clause in the Standard Reference text.

Delete any PSR clause which is not used or not relevant to the contract project.

SSCM Section 1		Miscellaneous Provisions

[Add changes to the existing clauses in this section here, in correct number order.]

SSRW Clause 1.3		Standards, Acts, and Publications

Note: In respect to Test Methods AS 1289.3.1.1, AS 1289.3.2.1, and AS 1289.3.4.1, Wet Preparation Method is to be used where this is an option in an applicable test method.

SSCM Section 1.4		Definitions

Sub-grade – [enter data]

[Add details regarding sub-grade for the project here OR refer to other clauses in these PSRs OR refer to other clauses in the specification.]

SSCM Clause 1.9		Establishment

SSCM Clause 1.9.1.1		General - Mobilisation

[Specify regional requirements if different from standard.]

SSCM Clause 1.14		Extraction Areas and Water Sources

SSCM Clause 1.14.9 		Rehabilitation of Material Extraction Areas

[Specify rehabilitation process.]

SSCM Clause 1.27		Project Notice Boards

Project Notice Board – [enter data]

[Indicate if Required or Not required

If required add details here and/or provide drawings]

SSCM Clause 1.29		Contractor’s Responsibilities and Submissions

[Add changes to the existing sub-clauses in this clause here, in correct number order.]

SSCM Clause 1.29.1.1 	Warranties

[Amend the required information for product warranties to suit the project.]

SSCM Clause 1.29.1.3 	Traceability

[Amend the required information for traceability of products to suit the project.]

SSCM Clause 1.29.1.4 	Warranties Schedule

| Table - Warranties Schedule |  |
| --- | --- |
| Clause Title | Section |
| [enter data] | [enter data] |
| [enter data] | [enter data] |
| [enter data] | [enter data] |
| [enter data] | [enter data] |

[Insert details for product warranties into table as appropriate.]

SSCM Clause 1.29.2	 	Certificates of Compliance

| Table - Certificate of Compliance Schedule |  |
| --- | --- |
| Clause Title | Section |
| [enter data] | [enter data] |
| [enter data] | [enter data] |
| [enter data] | [enter data] |

[Insert details for product certificates of compliance into table as appropriate.]

SSCM Clause 1.39		Other Requirements

[Add any additional requirements.]

SSCM Section 2 		Provision for Traffic

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 2.4		Temporary Traffic Management

[Add changes to the existing clauses in this section here, in correct number order.]

SSRW Clause 2.1		General

[If there are known alternative ways to deliver the requirements under the Contract make an assessment of whether any of those are acceptable, and if any of those are NOT acceptable. State clearly in the specification which alternatives are NOT acceptable.

Edit this worksection so that redundant working is deleted, and so that new requirements are added]

SSCM Clause 2.4.4		Site Based Workzone Traffic Management Designer (TMD)

Site based WZ1/TMD (Required/ Not required)	[enter data]

[Edit to suit the project – delete one option. Obtain Manager Road Operations and Traffic approval if site based WZ1/TMD is to be required.

If NOT required delete the rest of this sub-clause.]

[Consider the project requirement for an onsite Traffic Management Designer (to be positioned onsite for the contract duration. Considerations may be complex projects where regular changes are required to TTM which would otherwise result in delay to works.

If possible state the time period for which a TMD is required to be on site.

Delete if not applicable]

SSCM Clause 2.4.5.2		Traffic Pilot Vehicle – Other Requirements

Traffic Pilot Vehicle (Required/ Not required)		[enter data]

[Consider the anticipated conditions on site and select required/not required and add any conditions appropriate for the specific project. Delete sub-clause if escort/pilot vehicle not required Obtain Manager Road Operations and Traffic approval if Pilot vehicle is considered to be specified.

Consider requiring provision of a traffic pilot vehicle for work sites where criteria listed in AS 1742.3 and AGTTM Part 3 Roadworks pilot vehicle clause are present:

Consider requiring provision of a traffic pilot vehicle to guide traffic through a work site if any of the following conditions exist:

part of the length of the work site is out of view of the supervisor, workers and the traffic controller.

the traffic speed is required to be kept low to minimize damage to the works

traffic needs to follow a particular path through the site which may not be obvious unless a pilot vehicle is used.

the hazard to workers requires the traffic speed to be reduced.

pedestrians and cyclists require assistance in travelling through the work site and need to be driven to the end of the work site (rural roads).]

SSCM Clause 2.5		Traffic Management Plan

[State if Traffic Management audit is required or not required]

SSCM Clause 2.5.1		Document Control

Register of TMPs and TGSs for this project is –Required / Not required.

[Delete the redundant option. Do not require a project specific register for simple jobs where it is unlikely that revisions of TGSs will be required during the works.]

SSRW Clause 2.6		Compliance Checks of Worksite Traffic Management

[Do not advise the TMP Designer what the required changes are. Only advise what the identified hazards are]

SSCM Clause 2.7		Traffic Management Audit Requirements

The Traffic Management for this contract is;

- subject to audit

- not subject to audit

[Refer to the Traffic Management Plan clause in this worksection to see if audits are required for this contract. If audits are not required this clause can be deleted.]

SSCM Clause 2.8		Temporary Traffic Management Auditing Results

[Refer to the Traffic Management Plan clause in this worksection to see if audits are required for this contract. If audits are not required this clause can be deleted.]

SSCM Clause 2.12		Work In Urban/ Built-Up Areas

SSCM Clause 2.12.1		Working Times

Work within the restricted work hours in built up areas is permitted within the following limitations:

[Enter data]

[Modify the restricted working times to suit the site conditions. When determining restricted working hours consider traffic volumes in each direction, and any scheduled events or festivals that may affect the works. Restricted working hours must be advised at Pre-start meeting if it cannot be advised at tender stage.]

[Delete the clause if the work is not in an urban area unless high volumes of traffic are expected for an event. Include a cross-reference to the Restricted Working Hours clause in the Preliminaries if appropriate.]

SSCM Clause 2.12.2	Traffic Lanes

[Discuss the method of traffic control with the Client and Council as required. If required, insert a method of traffic control.]

SSCM Clause 2.14		Warning Devices

SSCM Clause 2.14.1		Works in Progress Signs

[Adjust text to suit the specific project]

SSCM Clause 2.15		NT Specific Directions for Road Works Signs

SSCM Clause 2.15.5		Portable Variable Message Signs (VMS)

[Include this clause where major disruptions or changes to the traffic path are likely to occur]

[Consider requirement such as; will the VMS only be required prior to commencement, and/or prior to opening to traffic (changed traffic conditions), or for the duration of the works. Also consider mobilization if outside of urban environment. Make requirements project specific and explicit. Add text to clearly detail requirements. Make explicit that DIPL can accommodate individual Contractor’s preferred methodology provided safety is not compromised.]

SSCM Clause 2.15.7		Temporary Speed Limits

Temporary speed limits applicable to the work site are:

| Table – Temporary Speed Limits |  |
| --- | --- |
| Zone/section of works | Speed limit(s) |
| [Enter data] | [Enter data] |
| [Enter data] | [Enter data] |

[If temporary speed limits stated in the Table – Target lowest speed limits are the only applicable limits, delete this table and clause. If there are no specific requirements delete this clause. Add more rows to the table if needed.]

[The speeds stated in the table are the lowest allowable speeds. Increase these minimums where required, eg. In a high traffic volume situation where lower speeds are not desirable.]

SSCM Clause 2.18		Detours, Side Tracks, and Crossovers

Detour/ Side Track/ Crossover required or not required;	[enter data].

[Determine with the Regional Director whether construction requires a detour or will be done under traffic. Delete as appropriate.

If detour required/permitted, provide as much information as possible including minimum lengths between detours, or how many detours can be under traffic at any one time i.e. one detour under traffic while another is constructed]

SSCM Clause 2.18.1		Design and Construction

[The design of detours must take in to account the local terrain where the detours will be located. The selected path of any detour must be such that all types and sizes of vehicles anticipated to use the detour will be able to use the detour without undue disruption to normal driving patterns. Special designs may be required to provide detours through restricted sites such as at jump-ups, through cuttings, around narrow built-up pavements, on flood plains, and other site conditions which may affect the provision of the detours.]

Provide side tracks for detours when it is impractical to provide for traffic on the existing road system.

Side Track Type;			[enter data]

[Specify either sealed or gravelled. Specify sealed side tracks when the detour will be in operation for longer than 4 weeks. Discuss the side track type and requirements with the Regional Project Officer. Delete if side track not required.]

Construct side tracks with a finished surface level crown height 250 mm above the natural surface.

[Delete this requirement for short duration projects on local roads which are unlikely to be affected by rain.

Ensure that primer sealing is included in the SPRAY SEALING Section.  Note here that the paragraph “Sealed side tracks to be primer sealed with 7 mm aggregate as specified in the SPRAY SEALING Section.” is deleted if it is not applicable.]

SSCM Clause 2.18.2		Maintenance

Prevent dust nuisance by water spraying at regular intervals to keep surface moist.

[Consider access to water and associated costs. If water supply is difficult consider deleting the requirement to use water.

Possible optional alternative is signage to warn road users of hazards from dust.]

SSCM Clause 2.19		Access to Adjacent Properties and Side Roads

| Table – Requirements for access to adjacent properties and side roads |  |
| --- | --- |
| Location | Requirements |
| [enter data] | [enter data] |
| [enter data] | [enter data] |

[Consider specifying access requirements for commercial properties. Negotiate with the owners for their access requirements during design. Identify the access requirements on the plans, if necessary. Add more rows if needed. Delete clause if there are no requirements.]

SSCM Clause 2.20		Temporary Pedestrian Access

| Table – Requirements for temporary pedestrian access |  |
| --- | --- |
| Location | Requirements |
| [enter data] | [enter data] |
| [enter data] | [enter data] |

[In situations where a temporary path is required, include an item in the Schedule of Rates outlining the standard required, e.g. "a temporary primer sealed and lit path". Add more rows if needed. Delete clause if there are no requirements.]

SSCM Clause 2.22		Contractor’s Plant and Equipment

Floodlight the road required or not required;	[enter data].

[Include only for roads carrying significant traffic]

SSCM Clause 2.26		Restoration

[Consider seeding with local native grasses and trees and shrubs where applicable.]

SSCM Clause 2.27		Other Requirements

[Add any additional requirements.]

SSCM Section 3 		Earthworks and Drainage

[Add changes to the existing clauses in this section here, in correct number order.]

SSRW Clause 3.3		Standards, Acts, and Publications

Note: In respect to Test Methods AS 1289.3.1.1, AS 1289.3.2.1, and AS 1289.3.4.1, Wet Preparation Method is to be used where this is an option in an applicable test method.

SSCM Clause 3.7		Fill Material

SSCM Clause 3.7.1		Standard Fill

| Table - Standard fill properties |  |  |
| --- | --- | --- |
| Property |  | Values required |
| CBR 4 day soaked at 95% MMDD to AS 1289: |  | [enter data] min. |
| Maximum Particle Size: | For subgrade layers | [enter data] mm |
|  | For other than subgrade layers | [enter data] mm |
| Plasticity Index: |  | [enter data]% - [enter data]% |

[Enter amended values if applicable to project.]

SSCM Clause 3.8		Removal of Excess Material

Haul and dump and/or spread excess material at these site(s): [enter data]

The material is to be:

- spread and sheeted with topsoil, or

- stockpiled.

[Specify requirements – delete items not required.]

SSCM Clause 3.9		Earthworks for Drainage

SSCM Clause 3.9.2		Table Drain Offlets

[Specify intervals for offlet drains if other than 150 m nominal.]

SSCM Clause 3.10 		Trim and Compact Unpaved Areas

[State shape and compaction required and any specified material property requirements. Indicate which verge areas are subject to this clause or call up a drawing which shows this information.]

SSCM Clause 3.13		Other Requirements

[Add any additional requirements.]

SSCM Section 4 		Conformance Testing

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 4.2		Standards, Codes, Test Methods, and Publications

Note: In respect to Test Methods AS 1289.3.1.1, AS 1289.3.2.1, and AS 1289.3.4.1, Wet Preparation Method is to be used where this is an option in an applicable test method.

SSCM Clause 4.9		Tables – Test Frequencies, Compliance Testing

Table – Test Frequencies for Soils, Parts 1 and 3.

Table – Testing and Reporting Completion Times – Part 1

Note: In respect to Test Methods AS 1289.3.1.1, AS 1289.3.2.1, and AS 1289.3.4.1, Wet Preparation Method is to be used where this is an option in an applicable test method.

SSCM Clause 4.10		Conformance Testing Results

For work in remote areas increase the testing and reporting completion times by a minimum of 2 days.

[Indicate if any work areas are remote.]

SSCM Clause 4.12		Other Requirements

[Add any additional requirements.]

SSCM Section 5 		Grading and Gravel Sheeting

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 5.15		Conformance

SSCM Clause 5.15.7		Exclusions to Specified Roughness Limits

[Indicate if any of the listed excluded areas must conform to an IRI of less than 2.4.]

SSCM Clause 5.16		Other Requirements

[Add any additional requirements.]

SSCM Section 6 		Stabilisation Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 6.6		In-Situ Stabilisation

SSCM Clause 6.6.1		Preparation of Subgrade Layer

[Add any additional requirements, eg; depth to be cut from top of base course layer, location of stockpile.]

SSCM Clause 6.10		Other Requirements

[Add any additional requirements.]

SSCM Section 7 		Spray Sealing Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 7.3		Standards and Publications

SSCM Clause 7.3.3		Other

Australian Asphalt Pavement Association (AAPA) has been renamed to Australian Flexible Pavement Association (AFPA).

SSCM Clause 7.5		Scope

SSCM Clause 7.5.1		Cycle and Pedestrian Shared Path Maintenance

[Add any further requirements for cycle and pedestrian shared path maintenance and references to drawings.]

SSCM Clause 7.7		Sprayers and Personnel

Australian Asphalt Pavement Association (AAPA) has been renamed to Australian Flexible Pavement Association (AFPA).

SSCM Clause 7.10		Binder Coat Requirements

SSCM Clause 7.10.2		Prime, Primer Seals and Enrichment Coats

Cut back requirements are:

Primer Seal:		AMC [enter data] to AMC [enter data]

[Generally AMC 2 to AMC 4.]

Enrichment Coat:	CRS [enter data]

[Generally CRS 170/60.]

SSCM Clause 7.10.5		Binder Coats, Tender Quantities

Spray rates:			[enter data]

[Show here the spray rates to be used as a basis for calculating tender quantities.]

SSCM Clause 7.12		Supply of Aggregate

[Use this section for aggregate supply contracts only.]

Supply and deliver aggregate into stockpiles at the following locations:

| Site | Aggregate size (mm) | Quantity (m3) |
| --- | --- | --- |
| [enter data] | [enter data] | [enter data] |
| [enter data] | [enter data] | [enter data] |
| [enter data] | [enter data] | [enter data] |

[Insert information for each stockpile site as appropriate.]

NEW ITEM

SSCM Clause 7.23.3	Price Adjustment for Bitumen – Basis of Adjustment

URL for Rise and Fall Indices has changed – now it is https://www.vic.gov.au/construction-works-and-services-tenders-and-contracts

END OF NEW ITEM

SSCM Clause 7.24		Other Requirements

[Add any additional requirements.]

SSCM Section 8 		Bituminous Surface Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 8.15		Conformance

SSCM Clause 8.15.6		Surface Roughness

[Indicate if required or delete if not required. If required state locations and roughness values to be achieved.]

SSCM Clause 8.19		Other Requirements

[Add any additional requirements.]

SSCM Section 9 		Concrete Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 9.9		Construction

SSCM Clause 9.9.3		Cycle and Pedestrian Shared Path Maintenance

[Add any further requirements for cycle and pedestrian shared path maintenance and references to drawings. Specify requirements for depth of concrete if other than 100 mm and specify reinforcement.]

SSCM Clause 9.20		Other Requirements

[Add any additional requirements.]

SSCM Section 10 		Drainage Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 10.6		Construction of Culverts and Structures

SSCM Clause 10.6.3 		Culverts in Fill under Construction

[Add design details and/or refer to drawings.]

SSCM Clause 10.13		Other Requirements

[Add any additional requirements.]

SSCM Section 11 		Protection Works Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 11.9		Dumped Rock Protection

Large spalls or boulders with least dimension of [enter data] mm.

[Specify size considering the availability of rock sizes.]

SSCM Clause 11.12		Gabions

SSCM Clause 11.12.2		Steel Wire Mesh for Gabions

[Specify here if PVC wire coating is not required.]

SSCM Clause 11.13		Reno Mattresses

SSCM Clause 11.13.2 		Steel Wire Mesh for Reno Mattresses

[Specify here if PVC wire coating is not required.]

SSCM Clause 11.17		Other Requirements

[Add any additional requirements.]

SSCM Section 12 		Road Furniture Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 12.7		Tactile Ground Surface Indicators

Table – TGSI Minimum Performance Standards – Colours [enter data]

[Specify colours of TGSIs if not already shown on drawings. Colours generally white, black, yellow, and/or blue.]

SSCM Clause 12.9.6		Plastic Flexible Guideposts - Materials

SSCM Clause 12.9.6.6		Delineators

For plastic flexible guide posts, use discrete device type retroreflectors to denote the following special hazards: [enter data]

[Indicate details of hazards here, or state that the hazards are on drawings.]

Attach discrete device type retroreflectors to manufacturer’s recommendations:[enter data]

[Specify where these can be used or refer to drawings. Delete if not applicable.]

SSCM Clause 12.10.6		Steel Flexible Guide Posts - Materials

SSCM Clause 12.10.6.6	Delineators

For steel flexible guide posts, use discrete device type retroreflectors to denote the following special hazards: [enter data]

[Indicate details of hazards here, or state that the hazards are on drawings.]

SSCM Clause 12.13		Road Signs – Manufacture, Supply, and Delivery – Materials

SSCM Clause 12.13.1		Anti-spear fixings for Hazard Markers (Sight Boards)

Delete existing sub-clause. Insert new sub-clause.

12.13.1			Anti-spear fixings for signs parallel to travelled path

Anti-spear fixings and stiffener rails must be installed for road signs which are installed parallel to the path of travel of traffic, and which have 3 posts.

The middle post is to be off-centre, on the departure side of the central splices for the rails. Refer to the plan view diagram below. Note; clamps and fixings not shown for clarity of diagram.

The stiffener rails are to be aluminium extrusions, each made up of two equal length sections, spliced at the centre line. The aluminium extrusions are to be of alloy and temper as shown on the drawings.

The aluminium extrusions are to be Capral EK8891S WA Type A, or Ullrich UA6490, or equal equivalent.

Use heavy duty clamps as shown on the drawings.

Refer to Civil Standard Drawings CS-3516, CS-3517, and CS-3518, accessible via https://dli.nt.gov.au/industry/technical-standards-guidelines-and-specifications/standard-drawings .

End of new sub-clause

SSCM Clause 12.13.3.5	Posts

Road signs mounting pole sizes other than the sizes shown in the Table - Roadside Signs - Mounting Selection are:

| Table - Roadside Signs – Non Standard Mounting Poles Selection |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location | Sign Size |  | Qty of posts | Bracing (Yes/ No) | Brackets or M8 bolts per post | Galvanised Post size |  |  | Footing |  |
|  | Width (mm) | Depth (mm) |  |  |  | NB (mm) | OD (mm) | Thickness (mm) | Diameter (mm) | Depth (mm) |

[Delete this clause and the table if no non-standard pole sizes are required.]

SSCM Clause 12.13.3.6	Anti-Graffiti Coating

Obtain Superintendent’s approval for the use of the anti-graffiti films or coating products. Apply anti-graffiti products only to the new road signs specified by the Superintendent.

[All roads Project Officers BE AWARE that 3M warrants their road signs only if their anti-graffiti film is used. Use of non-3M anti-graffiti product voids the 3M warranty.]

SSCM Clause 12.15		Road Signs – Installation and Maintenance

Replace the content of 12.5.5 and 12.15.6 with the content below.

SSCM Clause 12.15.5		Lateral Placement

Lateral placement to be measured to the edge of the sign nearest the road.

Lateral placement to be as follows:

Signs (frangible post used) on Unkerbed roads:

Two to 5m clear from the edge of the traffic lane, and 600 mm minimum clear from the outer edge of the road shoulder.

Signs (frangible post used) on Kerbed roads:

Preferably 500 mm to 1000 mm from the front face of the kerb.

The maximum clearance is 5.0m.

Rigid safety barrier present:

A minimum of 1.0 m from the face of the safety barrier (regardless of kerb position)

Wire rope safety barrier present:

A minimum of 1.5 m from the face of the safety barrier (regardless of kerb position)

SSCM Clause 12.15.6		Height

Height to be measured as the clearance above the nearest edge of the pavement to the lowest edge of the lowest sign in an assembly.

Heights for signs to be as follows:

| Table – Heights for signs |  |
| --- | --- |
| Unkerbed Roads |  |
| Signs overhanging a footway: | 2.5 m minimum above a defined pathway. |
| Fingerboard (G3) and street name signs (G5) and other navigational signs at intersections: | 2.0 m above the near edge of the pavement. |
| Signs at other locations: | 2.0 m above the near edge of the pavement in an environment with pedestrian movements |
|  | 1.0 m to 1.5 above the near edge of the pavement in areas not subject to pedestrian movements. |
| Kerbed Roads: |  |
| Signs overhanging a footway: | 2.5 m minimum above a defined pathway. |
| Signs not overhanging a footway: | 2.0 m clearance except for those specific signs on medians and islands given below. |

Table – Height of specific signs on medians and islands

The following signs, when used on medians and islands, to have a clearance 150 mm above the kerb:

D4-1-2 Hazard Marker

D4-2-2 Hazard Marker

D4-3 Hazard Marker

R2-3 (Keep Left) (Keep Right)

R2-5 (No U Turn)

R2-6 (No Right Turn) (No Left Turn)

R2-15 (U Turn Permitted).

End of new content.

SSCM Clause 12.22		Cattle Grids

[Add any additional requirements or details for repair and maintenance works.]

SSCM Clause 12.28		Other Requirements

[Add any additional requirements.]

SSCM Section 13 		Pavement Marking Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 13.16		Other Requirements

[Add any additional requirements.]

SSCM Section 14		Landscape Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 14.5		Extent of Work

[Insert a list of roads, road lengths and/or areas to be maintained.]

| Road Name / Identifier | Length / Section included in Contract |
| --- | --- |
| [enter data] | [enter data] |
| [enter data] | [enter data] |

[Delete this clause and the table if no roads to be maintained.]

SSCM Clause 14.13		Grass Cutting

SSCM Clause 14.13.1		Service levels

Cut grass on medians, verges and islands within the road reserve as follows:

| Table – Grass Cutting Service Levels |  |
| --- | --- |
| Maximum cut: | 50 mm from the ground. |
| Height not to exceed: | [enter data] |

[Insert required grass cutting heights for each area or road reserve if different from that shown in Table - Grass Height Specification in Grass Cutting clause in ROAD AND MARINE AMENITY MAINTENANCE.]

SSCM Clause 14.31		Other Requirements

[Add any additional requirements.]

SSCM Section 15		Slashing and Weed Control

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 15.12		Other Requirements

[Add any additional requirements.]

SSCM Section 16	Traffic Signals and Intelligent Transport Systems Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 16.39		Other Requirements

[Add any additional requirements.]

SSCM Section 17		Street Light Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 17.8		Materials

[Add any additional requirements or any alteration to the reference text.]

SSCM Clause 17.8.5		Control Equipment

[Edit to suit the project or delete if not required]

SSCM Clause 17.10		Excavation

SSCM Clause 17.10.3		Trenches

Nominal trench width: 600 mm.

Nominal trench depth: 1200 mm.

[Reflects a 12 m post top and 9 m single outreach.]

SSCM Clause 17.15		Existing Street Lighting

SSCM Clause 17.15.1		Disconnection and Removal

Deliver the salvaged materials to [enter data] .

[Provide address / location of shed / yard for delivery of salvaged items.]

SSCM Clause 17.6		Other Requirements

[Add any additional requirements.]

SSCM Section 18 		Street Sweeping

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 18.14		Other Requirements

[Add any additional requirements.]

SSCM Section 19	Aerodrome and Aeroplane Landing Area Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

[If the contract does not involve aerodromes this clause can be deleted.]

SSCM Clause 19.21		Audits - Aerodromes

SSCM Clause 19.21.3		Aerodrome Reporting Officer Inspections

Mandatory serviceability inspection: [enter data].

[Specify requirements.]

SSCM Clause 19.21.6		Key Performance Indicators

Table - Key Performance Indicators and Audit Categories Services Performance Plan – Part 1 of 2 parts

Attendance to inspections – ARO Inspections required: See 19.24.1 below.

SSCM Clause 19.24		Inspections - Aerodromes

SSCM Clause 19.24.1		Aerodrome Reporting Officer Inspections

Carry out scheduled inspections at the following frequencies:

| Table – ARO Inspections – Frequencies and Requirements |  |  |
| --- | --- | --- |
| Inspection | Frequency | Requirements |
| [enter data] | [enter data] | [enter data] |
| [enter data] | [enter data] | [enter data] |
| [enter data] | [enter data] | [enter data] |

[Specify inspection frequencies and requirements. Delete this sub-clause if not applicable.]

SSCM Clause 19.25		Inspections - ALAs

SSCM Clause 19.25.1		Serviceability Reporting Officer Inspections

Carry out scheduled inspections at the following frequencies:

| Table – SRO Inspections – Frequencies and Requirements |  |  |
| --- | --- | --- |
| Inspection | Frequency | Requirements |
| [enter data] | [enter data] | [enter data] |
| [enter data] | [enter data] | [enter data] |
| [enter data] | [enter data] | [enter data] |

[Specify inspection frequencies and requirements. Delete this sub-clause if not applicable.]

SSCM Clause 19.40		Other Requirements

[Add any additional requirements.]

SSCM Section 20		Bus Stop Maintenance

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 20.28		Other Requirements

[Add any additional requirements.]

SSCM Section 21		Road & Marine Amenity Maintenance

[Provide plans and lists of road and/or marine amenity areas to be maintained.

Add changes to the existing clauses in this section here, in correct number order.

If the contract does not involve either road or marine amenities this clause can be deleted.]

SSCM Clause 21.2		Outline Description

[List maintenance requirements and road and marine amenity areas in the contract.]

SSCM Clause 21.6		Reference Standard Drawings

[Provide locality map and list of road and marine amenity areas listed in the contract.]

SSCM Clause 21.8		Extent of Work

[Identify the extents of the road and marine amenity areas to be maintained here or on drawings.]

SSCM Clause 21.14	Maintenance of Road and Marine Amenity Area Toilets

SSCM Clause 21.14.1		Toilet systems

[List types of toilet systems and the locations of those toilets included in the contract.]

SSCM Clause 21.15		Graffiti

SSCM Clause 21.15.2		Graffiti Removal Activity Tables

For periodic maintenance contracts the minimum maintenance frequencies are: [enter data].

[Delete if contract is for Darwin region.]

SSCM Clause 21.16		Grass Cutting

SSCM Clause 21.16.1 		Service Levels for Grass Cutting

Special areas with grass heights other than those shown in Table – Grass Height Specification are:

| Table – Grass Height Specification – Special Areas |  |  |
| --- | --- | --- |
| Amenity | Maximum grass cut | Grass height does not exceed |
| [enter data]. | [enter data]. | [enter data]. |
| [enter data]. | [enter data]. | [enter data]. |
| [enter data]. | [enter data]. | [enter data]. |

[List and state special areas and associated grass heights. Delete if no special requirements.]

SSCM Clause 21.23		Provision of Firewood

SSCM Clause 21.23.2		Service Levels for Provision of Firewood

Nominate special areas for provision of firewood so that firewood stockpiles never fall below 200 kg: [enter data].

[Delete if no special areas.]

SSCM Clause 21.28	Cleaning and Maintenance of Boat Ramps and Barge Landings

[If the contract does not involve Boat Ramps and Barge Landings this clause can be deleted.]

SSCM Clause 21.28.2		Removal and Relocation of Sediment Build-Up

Sediment height level to trigger excavation in departure channel: [enter data].

Minimum tide level for use of the facility by members of the community: [enter data].

Transport all excavated material to the following disposal / nourishment area: [enter data].

[Delete if no dredging requirements.]

SSCM Clause 21.28.3		Boat Ramps and Barge Landings Concrete Deck

SSCM Clause 21.28.3.2	Maintenance Cleaning of Boat Ramps and Barge Landings Concrete Deck

Maintain the surface at the base of the boat ramp to enable the launching of boats at all times for tides above the minimum height of .[enter data]

[Indicate required tide height including the reference datum. Delete if this information is shown on the drawings.]

SSCM Clause 21.28.3.5	Cleaning and Maintenance of Boat Ramps and Barge Landings Activity Tables

For periodic maintenance contracts the minimum maintenance frequencies are: [enter data].

[Delete if contract is for Darwin region.]

SSCM Clause 21.28.5		Rock Armour

SSCM Clause 21.28.5.2	Maintenance of Loose or Displaced Rocks

Maintenance response time following notification of displaced rock hazard: [enter data].

[Delete if no rock armour.]

SSCM Clause 21.29.4	Cleaning and Maintenance for Jetties, Pontoons, Fishing Platforms and Filleting Tables Activity Tables

For periodic maintenance contracts the minimum maintenance frequencies are: [enter data].

[Delete if contract is for Darwin region.]

SSCM Clause 21.30.4	Maintenance for Marine Growth/Shellfish Encrustations on Structures Activity Tables

For periodic maintenance contracts the minimum maintenance frequencies are: [enter data].

[Delete if contract is for Darwin region.]

SSCM Clause 21.34		Other Requirements

[Add any additional requirements.]

SSCM Section 22 		Protective Coatings

[Add changes to the existing clauses in this section here, in correct number order.]

SSCM Clause 22.5	Protective Coatings

Special requirements for:

- Surface preparation:			[enter data]

- Primer:				[enter data]

- Site testing including reports:		[enter data]

[Add any special requirements. Delete if no special requirements.]

SSCM Clause 22.6	Protective Coating Specifications - Systems and Approved Products

[Add any additional requirements.

Specify which coatings are to be use and which components are affected. If inorganic zinc silicates are required specify them here.]

SSCM Clause 22.22		Other Requirements

[Add any additional requirements.]

SSCM Section 23 		Measurement and Payment

[Add changes to the existing clauses in this section here, in correct number order.

Specify items and materials in addition to those listed in the Schedule of Rates in the Response Schedules for the contract relevant to measurement and payment.

Specify any additional work or any changes to the SSCM relevant to measurement and payment.]

SSCM Clause 23.1		Miscellaneous Provisions

SSCM Clause 23.1.4		Establishment

SSCM Clause 23.1.4.1		Mobilisation

Mobilisation may not be measured separately and may be included in rates for applicable items.

[Specify mobilisation is measured separately or not.]]

SSCM Clause 23.9		Concrete Maintenance

SSCM Clause 23.9.2		Cycle/Footpaths

[Specify requirements for depth of concrete if more than 100 mm and specify reinforcement.]

SSCM Clause 23.25		Other Requirements

[Add any additional requirements.]

SSCM Section 24 		Referenced Australian Standards

[Add any additional requirements.]

Note: In respect to test methods AS 1289.3.1.1, AS 1289.3.2.1, and AS 1289.3.4.1, wet preparation method is to be used where this is an option in an applicable test method.

SSCM Section 25 		Other Referenced Authorities and Documents

Australian Asphalt Pavement Association (AAPA) has been renamed to Australian Flexible Pavement Association (AFPA)

[Add any additional requirements.]

SSCM Section 26 		Acts, Regulations, and Codes

[Add any additional requirements.]

SSCM Section 27 		Civil Standard Drawings for Civil Maintenance

[Add any additional requirements.]

SSCM Section 28 		Northern Territory Climate Zones Table

[Add any additional requirements.]